
POLICY - PROCEDURE
NEW SINGLE FAMILY RESIDENTIAL CONSTRUCTION

REQUIRED FINISHED FLOOR ELEVATION
1. Applicant/builder shall submit a proposed site plan/survey (plot plan) depicting a proposed Finished Floor Elevation (FFE) and lot corner grades along with application and construction plans.

a. This proposed site plan/survey (plot plan) shall be reviewed and compared to City approved development master drainage site plan which depicts all FFE and lot corner grade expectations.

i. If a master drainage plan does not exist, City staff shall verify consistency with minimum FFE requirements set forth by City Ordinance, Florida Residential Code and any associated Floodplain requirements.

b. Once the proposed site plan/survey (plot plan) has been reviewed and approved, the City of Titusville, Florida Approved stamp shall be applied to the document and included with the record permit and construction plans.

i. This City approved site plan/survey (plot plan) shall be delivered to the applicant/builder by whatever means appropriate.  The applicant/builder shall acknowledge receipt of the City approved site plan/survey (plot plan).  This may be acknowledged through email confirmation or signature on receipt of paper.  This acknowledgment of receipt from applicant/builder shall be included in the record permit file.

2. Applicant/builder shall form the slab/foundation to meet the approved FFE depicted on the City approved site plan/survey (plot plan).  

a. This City approved site plan/survey (plot plan) shall be the record document for new construction.  The City Engineer has either approved the proposed FFE depicted or revised the FFE to meet design standards.   This City approved site plan/survey (plot plan) shall always be included in the record file with the City of Titusville, Florida Approved stamp applied.  The applicant/builder shall be responsible to maintain a paper copy of this City approved site plan/survey (plot plan) on site for inspection purposes.

b. The applicant shall be responsible to construct the project according to this City approved site plan/survey (plot plan) document.

3. Applicant shall engage a certified surveyor to produce a form board survey (or foundation survey; depending on construction design) to verify the project is consistent with City approved site plan/survey (plot plan).

a. Applicant shall submit this form board/foundation survey to the City for review and approval.  This document may be hand delivered to our Department, mailed in, submitted to buildingpermits@titusville.com or directly to the record application file through the Citizenserve portal.

b. The form board survey shall be forwarded to the Building Official and reviewed to confirm consistency with the City approved site plan/survey (plot plan).

i. IMPORTANT: City staff shall not schedule a mono pre-pour or slab inspection until the form board survey has met approval by the Building Official or his designee.

4. Once the form board survey has met approval by the Building Official, the applicant may request the appropriate required inspection: Mono pre-pour for monolithic pour or Building pre-slab for floating slab associated with stem-wall construction.

a. Once the requested inspection has been scheduled, inspection staff shall verify form board/foundation survey has been submitted, reviewed and approved by the Building Official prior to conducting the requested inspection.

5. Once inspection staff has verified the form board/foundation survey has been submitted, reviewed and approved by the Building Official, the requested inspection shall be conducted.  
2/28/2019

Steve Adams CBO, CFM
Building Official, City of Titusville, Florida
